CEAM GoKart — Sheet Map Area

Full Project Documentation & Source Code

CEAM, MAHE Bengaluru  ·  Oct 2026

# 1. Project Overview

| Field | Value |
| --- | --- |
| Project | CEAM GoKart — Autonomous Driving on Sheet Track |
| Rover | rover1 — Raspberry Pi 5, ROS2 Jazzy |
| Camera | Intel RealSense D435I (USB 3, 640×480 @ 15 fps) |
| Current IP | 10.235.140.80 |
| Foxglove | ws://10.235.140.80:8765 |
| OS | Ubuntu 22.04 |
| Nav Stack | Nav2 — RegulatedPurePursuit controller |

The goal is to build a Bird's Eye View (BEV) map of the sheet track and navigate the rover autonomously via HTTP goal commands from a laptop — no SSH required during demos.

The track is a flat printed plastic sheet with road markings: white lane boundaries, yellow dashed centre lines, white crosswalk stripes, red STOP signs, white direction arrows, and a roundabout. White board walls form the arena boundary.

# 2. System Architecture

- Pi 5 rover running ROS2 Jazzy

- RealSense D435I mounted facing downward at ~62.6° tilt

- Nav2 (RegulatedPurePursuit) for autonomous navigation

- Foxglove Studio on laptop for visualization via WebSocket

- Flask goal server on rover port 5000 — HTTP commands from laptop

- static_transform_publisher for map→odom (no AMCL / no LiDAR)

# 3. Session History — What Was Done

## 3a. Sep 2026 — Nav2 Foundation (AB5 Area)

- Nav2 fully working on Pi 5 rover (Sep 11): map_server + AMCL + RegulatedPurePursuit

- SegFormer-b0 running at ~0.1Hz (seg_node.py), seg_costmap_node.py publishing /seg/costmap + /seg/obstacles

- Obstacle IDs: {0,1,9,14,20} = wall, building, fence, pole, traffic sign

- nav2_custom.launch.py created to avoid bringup_launch.py hardcoding issues

- Static transform approach discovered — bypasses AMCL entirely, rover moves straight

## 3b. Oct 4–6 — Nav2 Debugging

- All Nav2 nodes active for first time Oct 4; nav goal accepted; rover moved forward

- AMCL consistently failing to converge — covariance stuck at ~0.25

- Oct 6: Static transform (map→odom) confirmed working — rover moves straight, no spinning

- seg_layer detected bottle obstacle briefly

- collision_monitor crashing with invalid type error on FootprintApproach.points — fixed by removing points:1000, keeping only min_points:6

## 3c. Oct 8 — Track Area Move

- Moved to new sheet map arena (plastic printed track, white board walls)

- slam_toolbox installed; SLAM attempted but LiDAR map too noisy on flat plastic surface

- Decision: abandon LiDAR SLAM for this track, pivot to camera BEV

## 3d. Oct 13–14 — BEV Development (Depth-Based, USB 3 Fix)

- Lead confirmed: face camera down, use Lift Splat Shoot BEV

- Camera intrinsics confirmed: fx=607.57, fy=607.52, cx=323.53, cy=249.92 @ 640x480

- USB 2 → USB 3 cable swap: RealSense now at 5000M on Bus 003

- RGB stable at 14.75 FPS, aligned depth at 15.0 FPS after killing seg_node

- Camera calibrated via depth plane fit: height=0.153m, tilt=40.1°

- numpy downgraded to 1.26.4 in seg_venv (cv_bridge conflict with numpy 2.x)

- Depth decoded via np.frombuffer to bypass cv_bridge numpy conflict

- bev_node.py built: manual timestamp sync, BEST_EFFORT QoS — publishing at ~1-5 FPS

- BEV image visible in Foxglove but mostly black — depth too sparse near ground on USB 2.1

## 3e. Oct 17–19 — Homography BEV + Goal Server (Current Working State)

- SWITCHED to RGB homography BEV (bev_map_node.py) — depth approach abandoned for USB 2.1

- cv2.getPerspectiveTransform with 4 manually calibrated ground corners → 400×400 top-down canvas

- Key fix: publisher QoS MUST be BEST_EFFORT — Foxglove image panels reject RELIABLE topics

- Key fix: Foxglove Color mode MUST be RGB (Gradient renders black)

- BEV image now fully visible in Foxglove showing track surface ✅

- goal_server.py: Flask HTTP server on rover port 5000 — laptop sends curl commands

- Autonomous navigation confirmed working via curl goal commands

- Known issue: rover spins excessively — max_angular_vel needs reducing to 0.5

# 4. BEV Implementation

## 4a. Why NOT Depth-Based BEV

The D435I on USB 2.1 produces mostly invalid depth for near-ground pixels. The depth-based node (bev_node.py) gave a mostly black image — valid depth pixels were too sparse on the ground plane. bev_node.py is kept for reference only.

## 4b. Homography BEV — bev_map_node.py (ACTIVE)

RGB-only approach using cv2.getPerspectiveTransform. Four ground-plane corners in the 640×480 camera image are warped to a 400×400 top-down canvas. No depth data needed.

| Corner | Camera pixel (src) | BEV pixel (dst) |
| --- | --- | --- |
| Bottom-left | (160, 480) | (50, 380) |
| Bottom-right | (480, 480) | (350, 380) |
| Top-right | (540, 280) | (350, 50) |
| Top-left | (100, 280) | (50, 50) |

Camera: RealSense D435I, 640×480, tilt ~62.6°, height ~0.147m above track. Output canvas: 400×400 px, rgb8 encoding.

QoS: BEST_EFFORT on both subscriber and publisher. Foxglove panel: Color mode → RGB.

## 4c. Camera Calibration Parameters

| Parameter | Value |
| --- | --- |
| fx | 607.5739 |
| fy | 607.5168 |
| cx | 323.5252 |
| cy | 249.9227 |
| Resolution | 640×480 |
| Tilt angle | ~62.6° downward from horizontal |
| Height above track | ~0.147m |
| Distortion model | plumb_bob (d = [0,0,0,0,0]) |

# 5. Goal Server — Remote Navigation

Flask HTTP server on rover port 5000. Laptop sends curl commands — no SSH needed during demos.

| Method | Endpoint | Body | Action |
| --- | --- | --- | --- |
| GET | /status | — | Health check |
| POST | /goal | {"x":1.0,"y":0.5,"yaw":0.0} | Send single Nav2 goal |
| POST | /waypoints | {"points":[{"x":1,"y":0},{"x":2,"y":1}]} | Sequential waypoints (3s apart) |
| POST | /stop | — | Send goal (0,0,0) to stop |

Example commands from laptop:

curl http://10.235.140.80:5000/status

curl -X POST http://10.235.140.80:5000/goal \

-H "Content-Type: application/json" \

-d '{"x":1.0,"y":0.5,"yaw":0.0}'

curl -X POST http://10.235.140.80:5000/stop

If port 5000 is busy: fuser -k 5000/tcp

# 6. Nav2 Setup

| Component | Value |
| --- | --- |
| Global planner | NavFn (NavfnPlanner) |
| Local controller | RegulatedPurePursuit |
| Config file | ~/nav2_params.yaml |
| Map→odom | Static transform (no AMCL, no LiDAR) |
| Lifecycle manager | autostart:true in nav2_custom.launch.py |

Static transform command:

ros2 run tf2_ros static_transform_publisher \

--frame-id map --child-frame-id odom \

--x 0 --y 0 --z 0 --yaw 0 --pitch 0 --roll 0

# 7. Startup Sequence (Each Session)

SSH into rover first for every terminal:

ssh rover1@10.235.140.80

| Terminal | Command | Notes |
| --- | --- | --- |
| T1 | ros2 launch rover_description bringup.launch.py | Base robot description |
| T2 | pkill -f ydlidar | Kill LiDAR — not needed |
| T3 | ros2 launch realsense2_camera rs_launch.py rgb_camera.color_profile:=640x480x15 depth_module.depth_profile:=640x480x15 pointcloud.enable:=false align_depth.enable:=true | USB 3 cable required |
| T4 | ros2 launch ~/depthscan.launch.py | "No camera info" initially normal |
| T5 | pkill -f ekf_filter_node && sleep 2 && ros2 launch rover_description ekf_odom.launch.py | Kill duplicates first |
| T6a | ros2 run tf2_ros static_transform_publisher --frame-id map --child-frame-id odom --x 0 --y 0 --z 0 --yaw 0 --pitch 0 --roll 0 | Replaces AMCL |
| T6b | ros2 launch ~/nav2_custom.launch.py | Nav2 stack |
| T7 | ros2 launch foxglove_bridge foxglove_bridge_launch.xml | Connect at ws://10.235.140.80:8765 |
| T8 | source ~/seg_venv/bin/activate && python3 ~/bev_map_node.py | BEV node — DO NOT run seg_node.py |
| T9 | source ~/seg_venv/bin/activate && python3 ~/goal_server.py | Flask goal server |

Foxglove: Add Image panel → topic /bev/image → Color mode: RGB

# 8. Known Issues & Fixes

| Issue | Cause | Fix |
| --- | --- | --- |
| Foxglove "Waiting for image messages" | QoS mismatch — publisher was RELIABLE | Use BEST_EFFORT QoS in publisher |
| BEV all black | D435I USB 2.1 near-field depth invalid | Switch to RGB homography (bev_map_node.py) |
| Foxglove color wrong (dark) | Color mode set to Gradient | Change to RGB in panel settings |
| Port 5000 busy | Old flask server still running | fuser -k 5000/tcp |
| Rover spinning excessively | max_angular_vel too high | Reduce to 0.5 in nav2_params.yaml under FollowPath |
| CPU overload / camera drops | seg_node.py pegs CPU at 100% | Do NOT run seg_node.py or seg_costmap_node.py with BEV |
| RealSense 7-8 FPS | USB 2.1 bandwidth limitation | Use USB 3 Type-A-to-Type-C cable |

# 9. Pending Work

| # | Task | Status | Notes |
| --- | --- | --- | --- |
| 1 | Fix rover spinning | In progress | Reduce max_angular_vel to 0.5 in nav2_params.yaml |
| 2 | Whiteboard boundary detection | Not started | HSV white mask: H:0-180, S:0-50, V:200-255 → publish as Nav2 obstacles |
| 3 | Global track map | Not started | Options: hand-draw .pgm, SLAM, or stitch BEV frames |
| 4 | BEV → Nav2 costmap layer | Not started | Feed /bev/image into Nav2 local costmap as obstacle layer |
| 5 | Lift Splat Shoot BEV | Future | Lead recommendation — needs proper USB 3 depth pipeline |
| 6 | flask install check | Pending | pip install flask --break-system-packages on rover if not installed |

# 10. File Reference

| File | Status | Purpose |
| --- | --- | --- |
| bev_map_node.py | ACTIVE | RGB homography BEV — publishes /bev/image |
| goal_server.py | ACTIVE | Flask HTTP server for remote Nav2 goal delivery |
| bev_node.py | Reference only | Depth-based BEV — not working on USB 2.1 |
| nav2_custom.launch.py | ACTIVE | Custom Nav2 launch file |
| nav2_params.yaml | ACTIVE | Nav2 parameters (RegulatedPurePursuit) |
| depthscan.launch.py | ACTIVE | Depth image to laser scan |
| seg_node.py | Inactive | SegFormer-b0 semantic segmentation (kills CPU on Pi5) |
| seg_costmap_node.py | Inactive | Segmentation costmap publisher |

# 11. Source Code

## 11a. bev_map_node.py — RGB Homography BEV (ACTIVE)

#!/usr/bin/env python3

"""

BEV Map Node — CEAM GoKart Project, MAHE Bengaluru

RGB homography BEV — no depth needed.

Camera: RealSense D435I, 640x480, tilt ~62.6 deg

Publishes: /bev/image (rgb8, 400x400)

Run: source ~/seg_venv/bin/activate && python3 ~/bev_map_node.py

"""

import rclpy

from rclpy.node import Node

from rclpy.qos import QoSProfile, QoSReliabilityPolicy, QoSHistoryPolicy

from sensor_msgs.msg import Image

import numpy as np

import cv2

class BEVMapNode(Node):

def __init__(self):

super().__init__('bev_map_node')

# BEST_EFFORT required — Foxglove rejects RELIABLE image topics

qos = QoSProfile(

reliability=QoSReliabilityPolicy.BEST_EFFORT,

history=QoSHistoryPolicy.KEEP_LAST,

depth=1)

self.sub = self.create_subscription(

Image, '/camera/camera/color/image_raw', self.cb, qos)

self.pub = self.create_publisher(Image, '/bev/image', qos)

# src: 4 ground corners in 640x480 image — tune to your mount

src = np.float32([

[160, 480],  # bottom-left

[480, 480],  # bottom-right

[540, 280],  # top-right

[100, 280],  # top-left

])

self.OUT_W, self.OUT_H = 400, 400

dst = np.float32([

[50,  380], [350, 380], [350, 50], [50, 50],

])

self.H_mat = cv2.getPerspectiveTransform(src, dst)

self.get_logger().info('BEV map node ready — publishing /bev/image')

def cb(self, msg):

img = np.frombuffer(msg.data, dtype=np.uint8).reshape(

msg.height, msg.width, 3)

bev = cv2.warpPerspective(img, self.H_mat, (self.OUT_W, self.OUT_H))

cv2.circle(bev, (self.OUT_W//2, self.OUT_H-20), 8, (0,255,0), -1)

out = Image()

out.header   = msg.header

out.height   = self.OUT_H

out.width    = self.OUT_W

out.encoding = 'rgb8'

out.step     = self.OUT_W * 3

out.data     = bev.tobytes()

self.pub.publish(out)

def main():

rclpy.init()

node = BEVMapNode()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

finally:

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

## 11b. goal_server.py — Flask Goal Server (ACTIVE)

#!/usr/bin/env python3

"""

Goal Server — CEAM GoKart, MAHE Bengaluru

Flask HTTP server on rover port 5000.

Run: source ~/seg_venv/bin/activate && python3 ~/goal_server.py

If port busy: fuser -k 5000/tcp

"""

import threading, math

from flask import Flask, request, jsonify

import rclpy

from rclpy.node import Node

from rclpy.action import ActionClient

from nav2_msgs.action import NavigateToPose

from geometry_msgs.msg import PoseStamped

app = Flask(__name__)

ros_node = None

class GoalSender(Node):

def __init__(self):

super().__init__('goal_server')

self._client = ActionClient(self, NavigateToPose, '/navigate_to_pose')

def send_goal(self, x, y, yaw=0.0):

self._client.wait_for_server()

goal = NavigateToPose.Goal()

pose = PoseStamped()

pose.header.frame_id = 'map'

pose.header.stamp = self.get_clock().now().to_msg()

pose.pose.position.x = float(x)

pose.pose.position.y = float(y)

pose.pose.orientation.z = math.sin(yaw / 2)

pose.pose.orientation.w = math.cos(yaw / 2)

goal.pose = pose

self._client.send_goal_async(goal)

@app.route('/status', methods=['GET'])

def status():

return jsonify({'status': 'online', 'rover': 'rover1'})

@app.route('/goal', methods=['POST'])

def receive_goal():

data = request.json

ros_node.send_goal(data.get('x',0.0), data.get('y',0.0), data.get('yaw',0.0))

return jsonify({'status': 'goal sent'})

@app.route('/waypoints', methods=['POST'])

def waypoints():

points = request.json.get('points', [])

def run():

import time

for pt in points:

ros_node.send_goal(pt['x'], pt['y'], pt.get('yaw', 0.0))

time.sleep(3)

threading.Thread(target=run, daemon=True).start()

return jsonify({'status': 'waypoints sent', 'count': len(points)})

@app.route('/stop', methods=['POST'])

def stop():

ros_node.send_goal(0.0, 0.0, 0.0)

return jsonify({'status': 'stop sent'})

def spin_ros():

rclpy.spin(ros_node)

if __name__ == '__main__':

rclpy.init()

ros_node = GoalSender()

threading.Thread(target=spin_ros, daemon=True).start()

app.run(host='0.0.0.0', port=5000)

## 11c. nav2_custom.launch.py

from launch import LaunchDescription

from launch_ros.actions import Node

def generate_launch_description():

return LaunchDescription([

Node(package='nav2_controller', executable='controller_server',

output='screen', parameters=['/home/rover1/nav2_params.yaml']),

Node(package='nav2_planner', executable='planner_server',

output='screen', parameters=['/home/rover1/nav2_params.yaml']),

Node(package='nav2_behaviors', executable='behavior_server',

output='screen', parameters=['/home/rover1/nav2_params.yaml']),

Node(package='nav2_bt_navigator', executable='bt_navigator',

output='screen', parameters=['/home/rover1/nav2_params.yaml']),

Node(package='nav2_velocity_smoother', executable='velocity_smoother',

output='screen', parameters=['/home/rover1/nav2_params.yaml']),

Node(package='nav2_collision_monitor', executable='collision_monitor',

output='screen', parameters=['/home/rover1/nav2_params.yaml']),

Node(package='nav2_lifecycle_manager', executable='lifecycle_manager',

output='screen', parameters=[{

'use_sim_time': False,

'autostart': True,

'node_names': [

'controller_server', 'planner_server',

'behavior_server', 'bt_navigator',

'velocity_smoother', 'collision_monitor',

]

}]),

])

## 11d. depthscan.launch.py

from launch import LaunchDescription

from launch_ros.actions import Node

def generate_launch_description():

return LaunchDescription([

Node(

package='depthimage_to_laserscan',

executable='depthimage_to_laserscan_node',

name='depthimage_to_laserscan',

remappings=[

('depth', '/camera/camera/depth/image_rect_raw'),

('depth_camera_info', '/camera/camera/depth/camera_info'),

],

parameters=[{

'output_frame': 'base_link',

'range_min': 0.3,

'range_max': 4.0,

'scan_height': 10,

}],

ros_arguments=[

'--ros-args',

'--param', 'qos_overrides./camera/camera/depth/camera_info.subscription.reliability:=best_effort',

'--param', 'qos_overrides./camera/camera/depth/image_rect_raw.subscription.reliability:=best_effort',

]

)

])

## 11e. bev_node.py — Depth-Based BEV (Reference Only — Not Working on USB 2.1)

This was the original depth-based LSS-style BEV node. It is kept for reference. The depth approach was abandoned because the D435I on USB 2.1 produces too few valid ground-plane depth pixels, resulting in a mostly black BEV image. Use bev_map_node.py instead.

#!/usr/bin/env python3

import rclpy

from rclpy.node import Node

from rclpy.qos import QoSProfile, QoSReliabilityPolicy, QoSHistoryPolicy

from sensor_msgs.msg import Image

from nav_msgs.msg import OccupancyGrid

from cv_bridge import CvBridge

import numpy as np, cv2

class BEVNode(Node):

def __init__(self):

super().__init__('bev_node')

self.bridge = CvBridge()

self.latest_rgb = self.latest_depth = None

self.latest_rgb_stamp = self.latest_depth_stamp = None

self.last_proc_time = 0.0

self.frame_count = 0

# Calibrated extrinsics (Oct 14 2026)

self.cam_height   = 0.147   # metres

self.cam_tilt_deg = 62.6    # degrees downward

# Half-res intrinsics (320x240 after resize)

self.fx, self.fy  = 303.787, 303.759

self.cx, self.cy  = 161.763, 124.962

self.bev_res = 0.005; self.bev_width = 300; self.bev_height = 200

qos = QoSProfile(reliability=QoSReliabilityPolicy.BEST_EFFORT,

history=QoSHistoryPolicy.KEEP_LAST, depth=1)

pub_qos = QoSProfile(reliability=QoSReliabilityPolicy.RELIABLE,

history=QoSHistoryPolicy.KEEP_LAST, depth=1)

self.create_subscription(Image,

'/camera/camera/color/image_raw', self.rgb_cb, qos)

self.create_subscription(Image,

'/camera/camera/aligned_depth_to_color/image_raw', self.depth_cb, qos)

self.pub_img = self.create_publisher(Image, '/bev/image', pub_qos)

self.pub_bev = self.create_publisher(OccupancyGrid, '/bev/costmap', pub_qos)

def rgb_cb(self, msg):

self.latest_rgb = msg

self.latest_rgb_stamp = msg.header.stamp.sec + msg.header.stamp.nanosec*1e-9

self.try_process()

def depth_cb(self, msg):

self.latest_depth = msg

self.latest_depth_stamp = msg.header.stamp.sec + msg.header.stamp.nanosec*1e-9

self.try_process()

def try_process(self):

if self.latest_rgb is None or self.latest_depth is None: return

if abs(self.latest_rgb_stamp - self.latest_depth_stamp) > 0.5: return

now = self.latest_rgb_stamp

if now - self.last_proc_time < 0.2: return

self.last_proc_time = now

self.process(self.latest_rgb, self.latest_depth)

def process(self, rgb_msg, depth_msg):

rgb = self.bridge.imgmsg_to_cv2(rgb_msg, 'rgb8')

rgb = cv2.cvtColor(rgb, cv2.COLOR_RGB2BGR)

rgb = cv2.resize(rgb, (320, 240))

depth = np.frombuffer(depth_msg.data, dtype=np.uint16).reshape(

depth_msg.height, depth_msg.width).astype(np.float32)/1000.0

depth = cv2.resize(depth, (320, 240))

h, w = depth.shape

uu, vv = np.meshgrid(np.arange(w), np.arange(h))

Z = depth; valid = (Z > 0.05) & (Z < 4.0)

X_c = (uu - self.cx) * Z / self.fx

Y_c = (vv - self.cy) * Z / self.fy

tilt = np.radians(self.cam_tilt_deg)

ct, st = np.cos(tilt), np.sin(tilt)

Y_w = Y_c*ct - Z*st + self.cam_height

X_w = X_c; Z_w = Y_c*st + Z*ct

gnd = valid & (np.abs(Y_w)<0.30) & (Z_w>0.01) & (Z_w<self.bev_height*self.bev_res)

bev = np.full((self.bev_height, self.bev_width, 3), 30, dtype=np.uint8)

bx = ((X_w+self.bev_width*self.bev_res/2)/self.bev_res).astype(int)

by = (self.bev_height-1-(Z_w/self.bev_res)).astype(int)

ok = gnd&(bx>=0)&(bx<self.bev_width)&(by>=0)&(by<self.bev_height)

bev[by[ok], bx[ok]] = rgb[vv[ok], uu[ok]]

# ... (HSV segmentation and OccupancyGrid publishing omitted for brevity)

self.frame_count += 1

def main():

rclpy.init(); rclpy.spin(BEVNode()); rclpy.shutdown()

if __name__ == '__main__': main()